Competition Brief

Competition Title

Inclusive Skills:  Woodwork

Competition Overview

Competitors will make a plant pot stand. The timber will be precut and marked where the competitors will need to drill the holes for the dowels. The depth of the holes should be of uniform depth, markers on the drill bit are allowed. Key skills are hand eye co-ordination and accuracy in drilling. Health and safety considerations are

•Task duration: 2 hours

Entry Criteria

Competitors need to be Pre-entry, working towards entry level 1.

Entry Capacity Restrictions by organisation

Maximum of 1 student per location plus 1 reserve can be registered in case of absence.

Brief

- Competitors will follow instructions, (written, pictorial or bespoke), to construct a plant stand.

- The parts will be precut and will be marked where the competitor needs to drill. The drill bit can be marked to show the required depth using coloured tape, or bands. Competitors will put glue into the holes in the legs and then put a dowel into each hole. Glue can then be put into the holes at the ends of the runners and then the frame can be constructed.

- The plyboard top is precut and marked where each of the dome head nails go. Using a hammer and a nail holder, the top is attached to the frame. This does not form part of the marking criteria. (Support staff to assist in attaching the top if needed). Competitors may bring other nail holders if they prefer to do so, or other adapted tools.

Infrastructure List

- For those competitors visiting Coleg Elidyr all materials and tools will be supplied.

- A competitor needs to bring any personal protective equipment that they may wish/need to use.

- Each competitor will be allocated the following;

- 4 pieces of timber 500mm x 44mm x 44mm (legs)

- 8 pieces of timber 200mm x 44mm x 44mm (runners)

- Dowels

- Glue

- Dome head nails one top plyboard 300mm x 300mm

- Hammer

- Mallet

- 4 clamps and a vice

- Battery drill with a 10mm wood bit.

Please note that competitors should bring any other resources that they require as outlined in their statement of support.

Competition specific Rules

- Participants will receive a health and safety briefing.

- Participants will start, and finish work as instructed by the judges.

- All PPE to be provided by the competitor.

- All materials and tools will be supplied by the organisation, unless competitors wish to bring their own including adapted tools.

- Participants will have 2 hours to complete all their tasks.

- A statement of support outlining any individual needs of the learners will need to be submitted by the 20th February 2026.

- Participants must only receive support if it is highlighted in their support statement, unless it is to prevent an injury.

- All risk assessments will be provided to staff and learners before the competition commences.

Marking and Assessment

The marking and judging of this competition will be done by a team of experts from Further Education or industry champions using marking criteria and allocated marks to ensure consistency.

| A | Effective use of vice and tools | 25% |
| --- | --- | --- |
| B | Drilling Depth, neatness, correct use of components | 25% |
| C | Safe use of Hand Tools and effective use of PPE | 25% |
| D | Overall appearance of finished product | 25% |
|  | Total | 100% |

Competition Lead

Lead Contact:

Lewis Flindall

lewisflindall@colegelidyr.ac.uk

01550760479